TG12345 MkII Technical Information

The console is based on standard amplifier designs that are then altered for their specific use.

The console internal operating level is -10dBV

To get to tape there are a lot of transistors in between the mic input and the tape output!

Also take note that all the electronics is in the path all the time apart from the Limiter which can be switched in and out.

Mic transformer

Mic Amp F

Bandpass F1

Limiter

Presence Z

Bass Q

Fader buffer E

Pan/Bus Buffer B

If you go through a group then there is also:

Mix amp C1

Limiter

Fader buffer K

Presence Z

Bandpass F2

Bus buffer B

Mix amp C1

Fader Buffer K

Presence Z

Bandpass F3

Track Announce N

Output amp R

Output transformer (200R output impedance)

Mic Cassette

Mic Pre

This amplifier uses a version of the type A3 design redesignated as Type D.

C6 and R8 are repurposed as described in a technical note.

The gain is trimmed from 20-30dB in 0.5dB steps by resistance of 1K2 to 0R between the GC pin and 0V.

The main gain control is in fact attenuation between the input transformer and the Type D amplifier, However, the highest 3 gain settings are from three separate taps on the input transformer with higher turn ratios..

Filter (Band Pass)

The Bandpass Filter Type F1 is fed directly from the output of the Mic Pre.

The output of the band pass filter was followed by a 6U8 capacitor.

Limiter

The limiter design is fairly self explanatory, in the console it was fed from the bandpass filter via a bypass control.

EQ

Presence control type X amplifier. Inductances from the manual are basic showing just the total for each coil except for the first inductor that has the inductance of the first set of winding indicated. I have added the calculated inductance for each winding section that tallies close enough for the values.

Bass control Type Z

Line Output

Type R amplifier

Other Cassettes:

Line Input - Transformer balanced sometimes followed by Type G buffer.

Group cassette Type F2

Master Channel Type F3

Monitor section Type F4

Phase circuit uses Type P

Amplifier Types