QC Rack Audit

Session of 2026-04-19 / 2026-04-20

# 1. Executive Summary

This session closed out the manchild engine_v1 variant drift bug and built the scaffolding for a repeatable per-plugin QC harness. A new variant_drift rule, a diagnostic _targets breadcrumb on the engine, and a durable family/tier reference map were added. All work committed (ef1cd8c) and pushed to origin/master.

- manchild · engine_v1 → GREEN verdict, 0 findings, 41/41 presets applied.

- variant_drift QC rule added: declared-preset vs live engine-state diff with lastTarget breadcrumb.

- qc_family_map.md written: 73-family catalog, four-tier preset taxonomy, capability schema.

- Scope locked: audio effects only. MIDI / VSTi / audio-to-MIDI / multichannel spatial deferred.

- Git: commit ef1cd8c (29 files, 8099 insertions) pushed to origin/master.

# 2. The Bug

QC report 02-41-39 showed 2 M-S mode presets where thB drifted to ~0.265 regardless of preset value. Signature matched the silent variant drift anti-pattern: Orb loading a different engine module than the registry label claimed.

### Affected presets (pre-fix)

- M/S – Vocal Focus · thB=0.778 → engine.thB=0.260

- M/S – Side Control · thB=0.389 → engine.thB=0.265

# 3. Fixes Landed

## 3.1 Diagnostic breadcrumb — manChildEngine.v1.js

Added _targets object to capture the last-commanded threshold value, so variant_drift can distinguish "setter called with wrong value" from "DSP ramp did not settle".

const _targets = { thA: null, thB: null };

setThresholdA: (v) => { _targets.thA = v; P('thA').setTargetAtTime(v, audioCtx.currentTime, 0.01); },

setThresholdB: (v) => { _targets.thB = v; P('thB').setTargetAtTime(v, audioCtx.currentTime, 0.01); },

// getState() also returns: _thATarget, _thBTarget

## 3.2 variant_drift rule — qcAnalyzer.js

Surfaces the lastTarget alongside the declared-vs-live diff in drift findings.

const tKey = `_${pKey}Target`;

const tVal = (tKey in live) ? live[tKey] : undefined;

const tail = tVal === undefined ? '' : ` [lastTarget=${JSON.stringify(tVal)}]`;

## 3.3 Result

QC report 02-50-35 — Verdict: Looks good. 41/41 presets applied, 0 findings. Build SHA 627263f-dirty. The _targets breadcrumb remains as permanent diagnostic infrastructure.

# 4. QC Rack Architecture

## 4.1 Pipeline

- 1. Sweep: iterate over presets, apply each via engine.applyBulk, render pink-noise buffer.

- 2. Snapshot: capture engine.getState() + LUFS / peak / GR / spectral metrics per preset.

- 3. Analyze: run rule set (variant_drift + baseline checks) over snapshot array.

- 4. Report: Markdown verdict (Looks good / Not ready) + numbers table + findings with dev-notes.

## 4.2 Four-tier preset taxonomy

| Tier | Purpose | Applicability |
| --- | --- | --- |
| T1 | Universal defaults — bypass, unity, extreme min/max | Every plugin |
| T2 | High-value coverage — full parameter sweep at mid + extremes | Every plugin |
| T3 | Schema-conditional — only when capability flag is set (sidechain, freeze, LFO, MB, etc.) | Conditional |
| T4 | Pressure tests — sustained abuse, PRNG reproducibility, long-session drift | Conditional / opt-in |

## 4.3 Capability schema (paramSchema extension)

{ categories: string[], subcategories: string[], modes: string[] | null,

hasSidechain, hasFeedback, hasFreeze, hasLFO, hasStereoWidth,

hasMultiband, hasLookahead, hasTruePeak, hasPitchDetector,

hasLPC, hasFFT, hasWDF: boolean,

nonlinearStages: number, osThresholds: number[] | null,

latencySamples: number,

crossoverPhase: 'linear' | 'minimum' | null }

# 5. Family / Tier Map (current internal chart)

qc_family_map.md locks a 73-family catalog grouped by the six systems (Time / Tone / Dynamics / Character / Movement / Space) plus Pitch-Spectral, Restoration, Creative-FX, Neural-AI, Analysis, Composite, Instrument-specific, Specialized, Utility. Each family carries a required tier set.

Note: the internal catalog is DSP-implementation-granular (73 families) because QC checks attach to DSP primitives (FFT, WDF, LPC, lookahead, etc.), not to marketing categories. A coarser 13-family industry-aligned roll-up is proposed in Section 8.

# 6. QC Checks Catalog

| Rule ID | What it proves | Required for |
| --- | --- | --- |
| variant_drift | Declared preset == live engine state | All plugins (shipped) |
| mix_null | Dry leg + wet leg == input at Mix=0 | Any plugin with Mix |
| monosum_null | Width=0 collapses cleanly to mono | Stereo-width plugins |
| band_reconstruction | Sum of bands at unity == input | Multiband plugins |
| latency_report | Reported latency matches measured impulse delay | Lookahead / linear-phase |
| lpc_stability | ¦k_i¦ < 1 for all reflection coeffs | Formant / vocoder / Kelly-Lochbaum |
| wdf_convergence | Iteration converges within budget | Diode / fuzz / nonlinear WDF |
| fft_frame_phase | Phase coherence across frame boundary | Phase vocoder |
| neural_determinism | Same input + seed == same output | NAM / neural reverb |
| prng_seeded | Seeded PRNG reproducible run-to-run | Creative FX / glitch |

# 7. Deferred to Future Sessions

- MIDI effects (arpeggiators, note shapers, quantizers).

- Virtual instruments (VSTi) — sampler, synth, drum machine harnesses.

- Audio-to-MIDI transcription.

- Multichannel spatial (Atmos, Ambisonics, binaural).

- Workflow meta-plugins (routing, scene recall, macro hubs).

- Layer C userFix database — populated organically as plugins pass QC.

# 8. Internal-vs-Industry Chart — Do We Update?

The user proposed folding the 73-family internal catalog under a 13-family industry-aligned taxonomy:

- Dynamics

- Equalization (EQ / Filtering)

- Saturation / Distortion / Harmonics

- Time-Based Effects (Reverb / Delay)

- Modulation

- Pitch / Time Processing

- Stereo / Spatial Processing

- Analysis / Metering

- Restoration / Repair

- Creative / Glitch / Sound Design FX

- Mastering / Finalization Suites

- Channel Strip / Console Emulation

- Utility / Gain / Routing

## Recommendation

YES — add it as a parallel top-level axis, do NOT replace the 73-family catalog. The two serve different masters:

- 13-family industry chart = user-facing browse / marketing / pill grouping / store taxonomy.

- 73-family DSP catalog = QC tier assignment, capability schema, rule dispatch. Each industry category maps to N DSP families.

Concrete schema change: add `categories: string[]` (industry) AND `subcategories: string[]` (DSP families). The `categories` field is what UI/browse uses; `subcategories` drives QC. Example for Lofi Loofy: categories=["Saturation / Distortion / Harmonics", "Creative / Glitch FX"], subcategories=["bitcrush", "sample-rate-reduce", "chorus", "wow-flutter", "noise", "vinyl-sim", ...].

## Mapping sketch (13 industry → DSP families it covers)

| Industry category | DSP families from internal 73 |
| --- | --- |
| Dynamics | Comp, limiter, gate, expander, de-esser, transient, upward comp, MB-comp, MB-limiter, M-S comp |
| EQ / Filtering | Parametric EQ, graphic, shelving, dynamic EQ, linear-phase, tilt, filter, vowel filter, resonator |
| Saturation / Distortion | Tape, tube, transformer, diode WDF, soft/hard clip, bitcrush, fuzz, waveshaper, folder |
| Time-Based | Reverb (FDN, plate, spring, convolution, shimmer), delay (tape, BBD, digital, ping-pong, granular) |
| Modulation | Chorus, flanger, phaser, tremolo, vibrato, rotary, ensemble, auto-pan |
| Pitch / Time | Pitch shift, formant, auto-tune, harmonizer, time-stretch (PSOLA / PV), vari-speed |
| Stereo / Spatial | Width, M-S, Haas, stereo image, binaural, crossfeed |
| Analysis / Metering | Spectrum, goniometer, correlation, loudness (LUFS / K-sys), phase, transient |
| Restoration | De-noise, de-click, de-hum, de-crackle, de-reverb, spectral repair |
| Creative / Glitch FX | Granular, buffer repeat, stutter, reverse, glitch, freeze, bit-mangle, PRNG FX |
| Mastering suites | Composite — comp + EQ + limiter + imager chains |
| Channel strip | Composite — preamp + EQ + comp + gate per-channel |
| Utility / Routing | Gain, pan, phase-invert, mono-maker, trim, channel-split, side-chain router |

## Action items

- Update qc_family_map.md: add Section 3.5 "Industry roll-up axis (13)" with the mapping table above.

- Extend capability schema: rename current field to subcategories; add categories[] of 13-industry strings.

- Store/pill UI can switch to categories[] without touching QC dispatch logic.

- No QC semantics change — only presentation taxonomy.

# 9. Artifacts

- src/manChild/manChildEngine.v1.js — _targets breadcrumb + getState() exposure.

- src/qc-harness/qcAnalyzer.js — variant_drift lastTarget surfacing.

- memory/qc_family_map.md — 73-family catalog + tier taxonomy + schema.

- Downloads/qc_report_manchild_engine_v1_2026-04-20T02-50-35.md — green verdict proof.

- Git: ef1cd8c on origin/master.